Resilient Infrastructure Planning for GLOF-Prone Regions: Focus on the Western Himalayas and Kullu Valley

# Introduction

The retreat of mountain glaciers and the expansion of large glacial lakes are among the most striking and dynamic physical impacts of climate warming [1]. Coupled with shifts in the stability of surrounding rock and ice walls, the threat posed by glacial lake outburst floods (GLOFs) is progressively increasing over time, specially in Western Himalayan region [2], [3]. Increasing frequency of flood event from past two decades and inadequate river management practices are further intensifying the flood risks, resulting in severe impacts on lives, livelihood and critical infrastructure. As a recent example, in August 2024, persistent and intense rainfall caused cloudbursts and landslides across multiple districts in Himachal Pradesh, including Shimla, Kullu, and Mandi. It resulted in substantial loss of life and livelihood, as well as severe damage to critical infrastructures, including houses, bridges, schools, shops, and the road network were heavily affected. The Kullu-Manali highway, a vital transportation route suffered major disruption due to the damage from cloudbursts. During such times, rescue and emergency operations, specially evacuating the vulnerable population (such as the elderly, disabled, pregnant women and children under the age of five) is an urgency and needs to be carried out by emergency services (disaster force, evacuation team, ambulance) within 5-20 min to avoid any injury and loss of life. However, emergency services take hours and sometimes days to carry out these rescue operations. The key barriers for the emergency response teams include flooded surface transportation (road) networks, impassable roads, traffic congestion, and far-off located emergency service centers that are hard to reach in time under such conditions. Moreover, emergency responders rely on flood maps issued by District disaster management authority, which is formulated based on the aerial extent of inundation, which does not provide clear picture of the travel and evacuation route needs to be followed by emergency services. Further, there is a lack of proactive planning, such as the strategic stationing of emergency-response teams or centres.

# Objective

Proposed study aims to improve the preparedness of emergency services to GLOF, with the specific goal of improving their response time to no more than 20 minutes. We break down this into the following objectives:

- Objective-1: Local-level flood inundation. First, we want to perform flood inundation mapping at the local (municipal level); this involves assessing flood depth, flood extent and flow velocity for near-term (2030-50) and long-term (2050-2100) return periods. Based on this, we will categorise different regions from red (high prone to flooding) to green (least prone to flooding).

- Objective-2: Road vulnerability and vehicular mobility assessment. Here, we will assess the road vulnerability i.e., identifying traversable and blocked roads, which is needed for planning travel and evacuation routes for emergency vehicles (e.g., find quickest and safest routes).

- Objective-3: Emergency service location. Here, we will model the optimal and ideal location of emergency services and shelters for the flood-affected, to improve timely emergency response (Fig. 1). This will help in increasing the effectiveness of emergency responders and reduce loss of lives, in a more holistic manner.

By meeting the above objectives, we expect to bring down the response times for emergency services to under 20 minutes for GLOF (to be piloted in Kullu valley).

# Proposed methodology

The overall methodology is illustrated in Fig. 2; below we describe in detail how we will achieve our three objectives stated earlier: 1) Flood inundation mapping, 2) Road vulnerability assessment and identification of travel and evacuation routes, 3) Identifying optimal location of emergency service centers.

Objective 1: Flood inundation mapping

Meeting this objective involves deciding and gathering a series of data as input to the MIKE+ flood model, and then actually conducting the modeling.

Data gathering: Meteorological. Daily gridded rainfall data will be obtained from Indian Meteorological Department (IMD), which provides precipitation data at 0.25 ´0.25 degree (25 km´25 km resolution), for 122 years i.e., from 1901 to 2022.  Rainfall data for near (2024-2030, 2050) and long-term (2100) at 0.5 ´0.5 degree under RCP 4.5 and RCP 6 scenarios (future warming scenarios) will be obtained from World Climate Research Program Coordinated Regional Climate Downscaling Experiment (CORDEX) dataset for South Asia region [41]. Centre for Climate Change Research (CCCR)-Indian Institute of Tropical Meteorology is the nodal agency for modelling CORDEX data for South Asian region.

Data gathering: LULC and drainage analysis: Land use-Land cover (LULC) map for the study region will be obtained from LANDSAT satellite data, provided by NASA and US Geological Survey (USGS) [42]. Landsat 9, the latest series in Landsat mission of NASA will be utilized, which provides data at 15 m spatial resolution. It will be utilized to obtain information about built-up area, water bodies and other features of land. Daily discharge data for the study region will be obtained from Central Water Commission, India [43].

Data gathering: Terrain analysis. Digital elevation model will be created for assessing the elevation of flooded zone and other morphometric analysis such as nature of drainage pattern, topography, channel length, slope of terrain, drainage texture, pattern etc. DEM will be created using CARTOSAT-10m satellite data [44] and topographic map of the study region. CARTOSAT was launched on 5th May 2005 by Indian Space Research Organization (ISRO).  CARTOSAT dataset has been largely used for terrain modeling and morphometric analysis of different watersheds in India. CARTOSAT-10 m resolution data will be acquired from National Remote Sensing Centre, India [45].

Flood return period. Gumbel’s probability distribution (extreme value type-1 distribution) will be used to estimate the flood return period (one in 5, 10, 100 and 1000 years) and frequency [46].

MIKE+ flood modeling. MIKE + will be used for flood depth and extent mapping. Regions will be further classified as minor, moderate and major flooding regions based on the flood depth and flow velocity. MIKE+ is highly effective in flood management by identifying whether flooding is due to cloudbursts, extreme rainfall, and climate change, insufficient capacity in storm inlet, insufficient pipe conveyance capacity and volume, system failures, increased urbanisation, increased impervious surfaces and fast runoff [9]. The output from MIKE+ model (flood depth, flow velocity, flood extent) will be used to categorize different zones from red (highly prone to flooding) to green (least prone to flooding).

### Objective 2: Road vulnerability to floods and route planning

Open street map containing information about roads, railways, physical infrastructure will be downloaded from Survey of India, Department of Science and Technology, India [47]. Roads intersecting with flood depth will be identified for each flooding scenario by overlaying flood risk map with road network map in ArcGIS. Based on the information about flood water depth and flow velocity along different road networks linking emergency services location to shelter homes in Kullu, we will assess the safe driving speed and vehicle stability of emergency vehicles in the following manner:

Safe driving speed: It is considered as the maximum speed at which a vehicle can safely navigate through a flooded road. It varies as per different types of vehicles (2-wheeler, 3-wheeler, 4-wheller) and even as per different categories of four-wheelers (SUV, Sedan, Hatchback) etc. [34] established the depth-disruption function by taking into account flood water depth, probability of rainfall, vehicle speed, maximum speed limit allowed as per driving rules and regulations by transport authority [48]. Depth-disruption function was obtained by performing experiments on various vehicle types under different flood water depth conditions by [34]. It provides an empirical relationship between flood water depth and vehicle safe speed, which will be utilized in the proposed study to assess the safe driving speed of emergency vehicles during various flooding scenarios.

Vehicle stability: When the water depth surpasses the vehicle's wading capacity or the velocity of road water, it leads to severe damage and instability of vehicle i.e., vehicle cannot safely pass through that road section. 20 cm is the minimum water depth that can lead to vehicle instability [49], however, it further varies depending on the type of vehicle (small, medium, heavy duty vehicles). Threshold value indicates that it is risky to drive emergency vehicles above this water depth. [21] used 30 cm depth as threshold for England, [50] used 25 cm for Leicester, UK while [15] used 50 cm for New York. Data combined from various national guidelines, theoretical and experimental studies show that a small car, large passenger car and heavy duty vehicle will remain stable in the flood water with a depth of 0.3, 0.4 and 0.5 m and flow velocity less than 1 m/s [16], [51].

A threshold value for Indian scenario will be assessed for Kullu specific road conditions and type of emergency vehicles owned by emergency responders. Based on this, a road will be marked as “impassable” or “traversable”.  Blocked roads will be considered as impassable and accordingly the suggestions will be made for rerouting and relocating emergency service responders. Through this, we will identify the safe and quickest evacuation routes for emergency vehicles.